VPNAAS :

Activate the vpn service and the vpn in horizon from the globals.yaml

Download the VyOs image for the router

Create new VyOs Vm 512MB memory , generation 1 , 2 network adapter ( eth0 with openstack external and eth1 default switch ) ,4GB of disk space , download from the iso.

# Inside VyOS live session — install to disk install image (user vyos pwd vyos)

Configure

set interfaces ethernet eth0 address 192.168.200.50/24

set interfaces ethernet eth1 address 172.20.160.201/24

set protocols static route 0.0.0.0/0 next-hop 192.168.200.1

Commit

Save

run ping 192.168.200.116 # Must succeed — this is OpenStack's demo-router external IP run ping 172.20.160.100 # Must succeed — VM Host

Configure VyOS IPsec VPN :

set vpn ipsec interface eth0

set vpn ipsec ike-group OS-IKE-GROUP key-exchange ikev2

set vpn ipsec ike-group OS-IKE-GROUP lifetime 86400

set vpn ipsec ike-group OS-IKE-GROUP dead-peer-detection action restart

set vpn ipsec ike-group OS-IKE-GROUP dead-peer-detection interval 30 set vpn ipsec ike-group OS-IKE-GROUP dead-peer-detection timeout 120 set vpn ipsec ike-group OS-IKE-GROUP proposal 1 dh-group 14

set vpn ipsec ike-group OS-IKE-GROUP proposal 1 encryption aes256

set vpn ipsec ike-group OS-IKE-GROUP proposal 1 hash sha256

set vpn ipsec esp-group OS-ESP-GROUP pfs dh-group14

set vpn ipsec esp-group OS-ESP-GROUP lifetime 3600

set vpn ipsec esp-group OS-ESP-GROUP proposal 1 encryption aes256

set vpn ipsec esp-group OS-ESP-GROUP proposal 1 hash sha256

set vpn ipsec site-to-site peer OPENSTACK-PEER description OpenStack VPN Gateway

set vpn ipsec site-to-site peer OPENSTACK-PEER remote-address 192.168.200.116

set vpn ipsec site-to-site peer OPENSTACK-PEER authentication mode pre-shared-secret

set vpn ipsec site-to-site peer OPENSTACK-VPN remote-address 192.168.200.116
set vpn ipsec site-to-site peer OPENSTACK-VPN authentication mode pre-shared-secret
set vpn ipsec site-to-site peer OPENSTACK-VPN authentication pre-shared-secret L4BVPN-S3CR3T-2025

set vpn ipsec site-to-site peer OPENSTACK-VPN connection-type initiate

set vpn ipsec site-to-site peer OPENSTACK-VPN ike-group OS-IKE-GROUP

set vpn ipsec site-to-site peer OPENSTACK-VPN local-address 192.168.200.50

set vpn ipsec site-to-site peer OPENSTACK-VPN tunnel 1 esp-group OS-ESP-GROUP

set vpn ipsec site-to-site peer OPENSTACK-VPN tunnel 1 local prefix 172.20.160.0/24

set vpn ipsec site-to-site peer OPENSTACK-VPN tunnel 1 remote prefix 10.0.0.0/24

Commit

Save

In openstack :

- IKE POLICY :

openstack vpn ike policy create OS-IKE-POLICY \

--ike-version v2 \

--auth-algorithm sha256 \

--encryption-algorithm aes-256 \

--phase1-negotiation-mode main \

--pfs group14 \

--lifetime units=seconds,value=86400

- IPseC POLICY :

openstack vpn ipsec policy create OS-IPSEC-POLICY \

--auth-algorithm sha256 \

--encryption-algorithm aes-256 \

--encapsulation-mode tunnel \

--transform-protocol esp \

--pfs group14 \

--lifetime units=seconds,value=3600

- VPN SERVICE :

ROUTER_ID=$(openstack router show demo-router -f value -c id) SUBNET_ID=$(openstack subnet show demo-subnet -f value -c id)

openstack vpn service create VPN-SERVICE \

--router $ROUTER_ID \

--subnet $SUBNET_ID

openstack vpn endpoint group create LOCAL-EG \

--type subnet \

--value $SUBNET_ID

openstack vpn endpoint group create REMOTE-EG \

--type cidr \

--value '172.20.160.0/24'

openstack vpn ipsec site connection create SITE-CONN-VYOS \

--vpnservice VPN-SERVICE \

--ikepolicy OS-IKE-POLICY \

--ipsecpolicy OS-IPSEC-POLICY \

--peer-address 192.168.200.50 \

--peer-id 192.168.200.50 \

--psk 'L4BVPN-S3CR3T-2025' \

--initiator response-only \

--peer-cidr 172.20.160.0/24

# NOTE: OpenStack is "response-only", VyOS "initiates". This is important.

SECURITY GROUP :

openstack security group rule create $SG_ID \

--protocol icmp \

--remote-ip 172.20.160.0/24 \

--description "Allow ping from VPN LAN"

openstack security group rule create $SG_ID \

--protocol tcp \

--dst-port 22 \

--remote-ip 172.20.160.0/24 \

--description "Allow SSH from VPN LAN"

# On the OpenStack host itself (or any VM on 172.20.160.0/24 you want to test from)

sudo ip route add 10.0.0.0/24 via 172.20.160.201

VERIFY :

# On VyOS

Run show vpn ipsec sa

# Should show an SA with your peer 192.168.200.116, state "up"

run show vpn ipsec status

# Should show "1 up" if tunnel is established

run show vpn ike sa # Shows IKE SA details

OPENSTACK :

openstack vpn ipsec site connection list # Column "Status" should be ACTIVE (not DOWN or ERROR)

# Look inside the router namespace for the running strongSwan

ip netns list | grep qrouter

# Then:

ip netns exec qrouter-<YOUR-ROUTER-ID> ip xfrm state list

# Should show SPIs and crypto transforms

ip netns exec qrouter-<YOUR-ROUTER-ID> ip xfrm policy list

# Should show policies matching 172.20.160.0/24 <-> 10.0.0.0/24

# From VyOS:

run ping 10.0.0.238 source-address 172.20.160.201

# This is the most direct test — VyOS LAN → VPN → web-1

# From OpenStack host (after adding route): ping 10.0.0.238

# Inside web-1 (if you have console access):

# Check what the source IP of arriving packets is:

sudo tcpdump -i any icmp

# You should see ICMP arriving from 172.20.160.x

Debug and errors  :

4 root causes were found and fixed:

Issue 1 — CLI flag conflict. --peer-cidr and --local-endpoint-group are mutually exclusive. You have to pick one mode: either use endpoint groups for both local and peer, or drop them entirely and use --peer-cidr inline.

Issue 2 — PSK mismatch. The pre-shared key was different on OpenStack vs VyOS (wrong characters, wrong case). PSK is case-sensitive and must be byte-for-byte identical.

Issue 3 — Traffic selector mismatch. OpenStack's --peer-cidr was 10.10.10.0/24 but VyOS's actual LAN was 172.20.160.0/24. IPsec selectors must match exactly or the SA negotiation fails silently.

Issue 4 — Wrong VPN driver (the big one). All three Kolla config files referenced OpenSwanDriver, which is deprecated and non-functional with modern strongSwan. This caused the VPN service to stay DOWN and the connection to hang at PENDING_CREATE indefinitely. The fix was replacing it with StrongSwanDriver in all three files and restarting the container.

Step 2 — Fixed the Kolla VPN driver by editing all three config files on the host to replace OpenSwanDriver with StrongSwanDriver:

sudo sed -i 's|...OpenSwanDriver|...StrongSwanDriver|g' /etc/kolla/config/neutron/l3_agent.ini

sudo sed -i 's|...OpenSwanDriver|...StrongSwanDriver|g' /etc/kolla/config/neutron/neutron_vpnaas.conf

sudo sed -i 's|...OpenSwanDriver|...StrongSwanDriver|g' /etc/kolla/neutron-l3-agent/l3_agent.ini

docker restart neutron_l3_agent

This moved the VPN service from DOWN to ACTIVE and the connection from PENDING_CREATE to DOWN.

That last step — correct PSK + correct CIDR + VyOS initiating — flipped the connection to ACTIVE.

If tunnel stays DOWN ::

# On VyOS — force initiate:

run reset vpn ipsec-peer 192.168.200.116

# Check logs: run show log vpn ipsec

# Look for IKE_INIT exchanges, AUTH failures, or CHILD_SA proposal mismatches

# On OpenStack — check VPNaaS agent logs:

docker logs neutron_vpnaas_agent --tail 100

# Look for "Connection ... became ACTIVE" or proposal mismatch errors

# Common issue: mismatched proposals — check IKEv2 parameters on BOTH sides match exactly # aes256 + sha256 + dh-group 14 on both sides

Annexe :

vpn_device_driver=neutron_vpnaas.services.vpn.device_drivers.strongswan_ipsec.StrongSwanDriver

Checklist for new VPNaaS deployments on Kolla1globals.yml must have

1 enable_neutron_vpnaas: "yes" before deployment

2 Set vpn_device_driver = StrongSwanDriver in all

3 Kolla config files3Verify ipsec version inside neutron_l3_agent container (strongSwan must be present)

4 PSK must be identical character-for-character on both sides (case-sensitive)

5 OpenStack peer-cidr must match VyOS tunnel local prefix exactly

6 IKE/IPsec proposals must match: aes256 + sha256 + dh-group14 on both sides

7 If OpenStack is response-only, VyOS must have connection-type initiate

8 Debug order: check l3_agent logs → ipsec.conf → ipsec.secrets → VyOS show vpn ipsec sa

9 Generated ipsec config lives at /var/lib/neutron/kolla/ipsec/<router-id>/etc/10After any config file edit,

always docker restart neutron_l3_agent and recheck